2026 NEC Correspondence (Part 2)

Identify the choice that best completes the statement or answers the question.

Chapter 5

- Things not covered under Hazardous (Classified) Locations, are features of equipment that involve _____ potential sources of ignition.

- fire		                                     		c.   nonelectrical

- shock                                                      	d.   hazardous

Article

- In Hazardous (Classified) Locations, gas detection equipment shall be _____ for Class I, Division 1 and for detection of the specific gas or vapor to be encountered.

- listed                                             		c.   marked

- labeled                                                        	d.   manufactured

Article

- A Class I, Division 1 location is a location in which ignitable concentrations of flammable gases, flammable liquid- produced vapors, or combustible liquid- produced vapors can exit under _____ operating conditions.

- adverse                                               	c.  adequate

- abnormal                                             	d.  normal

Article

- A Class II, Division 1 location is a location in which combustible_____ is in the air under normal operating conditions in quantities sufficient to produce explosive or ignitable mixtures.

- gas                                                		c.   dirt

- dust 	                                      		d.   fiber

Article

- Locations where nonmetal combustible fibers/flying are in the air under normal operating conditions in quantities sufficient to produce explosive mixtures is considered a _____ location.

- Class II, Division 2	                            	c.   Class III, Division 1

- Class I, Group A                                        	d.   Class III, Division 2

Article

- The specific bonding means permitted in Hazardous (Classified) Locations include, connections made up wrench tight using threaded couplings, threaded entries, or_____ threaded hubs on enclosures.

- approved                                                  	c.   labeled

- field                                                      		d.   listed

Article

- In Hazardous (Classified) Locations, an_____ bonding jumper is not required in Class I, Class II, or Class III, Division 2 locations where three provisions are met.

- auxiliary                                             		c.  additional

- equipment                                               	d.  specific

Article

- Reconditioned equipment is permitted to be installed in Class I locations and any changes to the _____ protection features of reconditioned equipment shall be assessed as a part of the reconditioning process.

- explosion                                           	c.   hazardous

- corrosion                                               	d.   flammable

Article

- Reconditioned equipment in Class I locations, are required to be listed or _____ for use in hazardous(classified) locations.

- marked                                                         	c.   manufactured

- labeled                                            		d.   identified

Article

- Which table in Article 501 covers wiring methods with flexible connections for use in Class I, Division 2 Locations.

- Table 501.10(B)(1)                                       	c.  Table 501.10(A)(1)

- Table 501.10(B)(2)                                   	d.  Table 501.10(A)(2)

Article

- In Class I, Division 1 locations, where the seal is _____ located on the Division 1 wiring method shall extend to the seal.

- clearly					c.   not

- definitely                                                       	d.   openly

Article

- In Class I locations, the modification of luminaires, using retrofit kits _____ for the location and listed for field installation with the luminaires to be retrofitted, shall be permitted.

- listed                                                           	c.   marked

- labeled                                                       	d.  manufactured usually

Article

- Reconditioned equipment in Class II locations, shall have any changes to the explosion protection features _____ as a part of the reconditioning process.

- evaluation                                                 	c.   required

- diagnostics                                                    	d.  assessed

Article

- Table _____ covers wiring methods in Class II, Division 1 locations.

- 502.10(A)(2)                                               	c.   501.10(A)(1)

- 502.10(A)(1)                                             	d.   502.10(B)(1)

Table

- Luminaires in Class II locations shall be permitted to be modified using listed _____ kits listed for the location for field installation.

- retrofit                                                        	c.   factory

- reconditioning                                              	d.   modification

Article

- Class II locations _____ permit any changes to the explosion protection features of the reconditioned equipment shall be assessed as a part of the reconditioning process.

- shall not                                                      	c.   shall

- may	                                                   	d.   may not

Article

- Type TC cable in Class III, Division 1 locations are covered in _____.

- Table 503.10(A)(2)                                     	c.   Table 503.10(A)(3)

- Table 503.10(A)(1)                                     	d.   Table 503.10(B)(1)

Table

- Class III, Division 1 and 2 luminaires using retrofit kits is covered in _____.

- 503.130                                                          	c.   503.130(A)

- 503.130(D)                                                    	d.  503.130(E)

Article

- In a intrinsically safe system a simple apparatus shall be permitted to be installed in any Class I, Division 1 and Class I, Zone _____ locations in which the maximum surface temperature of the simple apparatus does not exceed the ignitions temperatures of the combustible dusts or ignitable fibers/ flying present.

- 20, 21, and 22                                             	c.   20 and 21

- 0 and 1                                                           	d.   0, 1, and 2

Article

- The gas detection equipment used shall be listed for Zone 1 locations and listed for the detection of the _____ or vapor to be encountered.

- gas                                                              	c.   smell

- specific gas 	                                     	d.   cloud

Article

- Reconditioned equipment shall be permitted in Zone 0, 2, and 2 locations as specified in 505.3 _____ and 505.3 _____.

- (A)(1), (B)(1)                                                 	c.   (A)(2), (B)(2)

- (B)(1), (B)(2)                                               	d.   (A)(1), (A)(2)

Article

- A new protection technique for Zone 2 is _____ ventilated room “v”.

- manually                   			c.  positively

- artificially                                        		d.  negatively

Article

- Modification of luminaires, using retrofit kits listed for Zone 0, 1, and 2 locations listed for field installation _____ permitted.

- shall be 	                                  		c.   may be

- shall not be                                  		d.   can be

Article

- The wiring methods of 504.20, in Zone 1 and 2 shall be used for intrinsically safe circuits “_____” or “_____”.

- m, d                        				c.   ma, db

- ia, ib                                              		d.   e, eb

Article

- In Zone 1, for artificially ventilated rooms, conduit seals shall be provided in _____.

- one                                                             	c.   all

- several                                                          	d.   each

Article

- Flexible cables in Zone 2 _____ permitted to be installed in accordance with 505.17(C)(1) and 505.17(C)(2).

- may be					c.   shall be

- shall not be		                         	d.   can be

Article

- Specific bonding means in Zone 0, 1, and 2 hazardous(classified) locations, shall not permit, locknuts- bushings and double- locknut types, bonding locknuts used around impaired connections, such as reducing washers or oversized, concentric, or _____ knockouts.

- oblong                  				c.   non-standard

- eccentric 					d.   factory

Article

- Equipment that is listed for Zone 20 shall be _____ in a Zone 21 or Zone 22 location of the same combustible dust, combustible fiber/Flying, or ignitable fiber/Flying.

- permitted 			           		c.   allowed

- installed                                          		d.   certified

Article

- In Zone 20, 21, and 22 locations, equipment requirements now allow modifications of luminaires, using _____ listed for the location listed for field installation with the luminaire to be retrofitted.

- parts                                                         	c.  equipment

- luminaires                                             	d.  retrofit kits

Article

- The wiring method for Type TC-ER-HL cable in Zone 21 locations is listed in Table_____.

- 506.15(C)(1)			           	c.   506.15(A)(2)

- 506.15(B)(1) 		                        	d.   506.15(A)(1)

Article

- Zone 20, 21, and 22 locations, shall not have uninsulated exposed parts such as electrical conductors, buses, terminals, or components, that operate at more than _____ volts (_____ volts in wet locations).

- 30, 15                                                         	c.  240, 120

- 120, 60                                                       	d.  480, 240

Article

- Equipment bonding jumpers in Zone 20, 21, and 22 locations using LFMC _____ include an equipment bonding jumper of the wire type in accordance with 250.122.

- shall not					c.   shall

- may		                                      	d.   may not

Article

- Electric Vehicle Power Transfer Systems located within Class I locations in a commercial garage as defined in 511.5 shall _____ electric vehicle connections within that location.

- allow                                                            	c.   have

- not allow                                               	d.   permit

Article

- Cannabis oil preparatory equipment, cannabis oil extraction equipment, cannabis oil post-processing equipment, cannabis oil booths, and cannabis oil systems shall be ____ for the location.

- listed 			           		c.   permitted

- labeled                                            		d.   allowed

Article

- Cannabis extraction _____ be located in any building containing assembly, educational, day care, health care, ambulatory health care, residential board and care, residential, or detention and correction facilities.

- shall                                                    		c.  can be

- will                                             		d.  shall not

Article

- All ungrounded wiring in aircraft hangers, where such wiring is located in vaults, pits, or ducts, adequate drainage shall be provided and shall comply with the requirements for _____ locations.

- Class II, Division 1	           			c.   Class III, Division 2

- Class I, Division 2	                                     	d.   Class I, Division 1

Article

- Systems in health care facilities, shall have the capacity and rating to meet the _____ actual demand likely to be produced by the connected load on the system.

- maximum                 			c.   spike in

- minimum                   			d.   amount of

Article

- Wiring and protection, in health care facilities shall not apply to _____ used exclusively for psychiatry, alternative medicine, or optometry.

- rooms                                                           	c.  areas

- spaces                                                          	d. offices

Article

- The branch circuit serving life safety lighting and power circuits _____ be part of a multiwire branch circuit.

- shall                                                      		c.  shall not

- are permitted to                                           	d.  may

Article

- In health care facilities, ground-fault protection of equipment with automatic disconnecting means _____ be required between on-site power sources or set of power sources and any essential electric system transfer switch, or on the load side of any essential electrical system transfer switch.

- shall not                                                     	c.  may be

- shall                                                        		d.  can be

Article

- Receptacles and fixed equipment used in wet procedure locations of health care facilities, special protection shall be provided for a power distribution system in which the power supply is interrupted if the ground-fault current does, in fact, exceed the trip value of a Class ______ GFCI.

- B                            				c.  A

- D                              				d.  E

Article

- Essential electrical system shall have a _____ placed at the service-entrance equipment, indicating type and location of each on-site power source.

- arrow                                                       	c.   marker

- sign                                                 		d. luminaire

Article

- A warning sign shall be _____ where removal of a grounding or bonding connection in a power source equipment interrupts the grounding electrode conductors connection to another power source(s) grounding connection.

- labeled                                       		c.   indicated

- marked                                                   	d.  installed

Article

- Type 1 essential electrical systems _____ permitted to serve Category 2, Category 3, and Category 4 spaces.

- shall be                                       		c.   will be

- shall not                                                   	d.  may be

Article

- In Health Care Facilities, Utility Supply Power shall not be used as a source for the EES unless _____ elsewhere in this article

a. allowed                                                          	c.   indicated

b. permitted                                                 	d.   available

Article

- The connections to the Life Safety Branch shall be _____ to circuits essential to life safety.

- permitted                                       		c.  connected

- allow                                            		d.  limited

Article

- Illumination for means of egress shall have switching arrangements to _____ patient corridor lighting in hospitals from general illumination circuits to night illumination circuits shall be permitted is only one of two circuits can be selected, and both circuits cannot be extinguished at the same time.

a. connect                                                       	c.   switch

b. transfer                                                         	d.   disconnect

Article

- Required power sources for EES _____ shall be served by two or more independent sources or sets of power sources and sets of feeders designed to ensure sufficient reliability for effective facility operation consistent with the facility’s emergency operations plan.

a. needs                         				c.   loads

b. services                                         		d.   systems

Article

- Essential Electrical systems shall be separated from the life safety branch and equipment branch and raceways, cables, or enclosures for life safety and critical branches shall be _____ identified as components of the EES.

a. readily                                                        	c.   purposely

b. distinctly                                                         	d.   factory

Article

- OCPDs serving the EES shall be coordinated for a period of time that a fault’s duration extends beyond _____.

- 1 minute                                                        	c.   0.01 second

- 2 seconds                                          		d.   0.1 second

Article

- Generator accessories in life safety supply powers include, but not limited to, the transfer fuel pump, electrically operated for louvers, and other generator accessories essential for generator operation arranged for _____ connection to the on-site power source.

a. automatic                                     		c.   time-delayed

b. non-delayed-automatic 			d.   delayed

Article

- In patient care spaces using systems less than 120 volts, nominal such as Class 2 lighting circuits cabling fed from critical or life safety branch power shall comply with the mechanical protection requirements of Article_____.

- 517.81                                                          	c.  517.80

- 517.82                                            		d.  517.166

Article

- The isolated power systems in health care facilities, such as, line isolation monitor (LIM) circuits _____ tested after installation and prior to being placed in service.

- must be                                                         	c.   will be

- shall be                                                       	d.   can be

Article

- The temporary wiring in exhibition halls used for display booth, as in trade shows, flexible cables and cords approved for hard or extra-hard usage shall be _____ to be laid on floors where protected from contact by the general public.

- permitted  	                                       	c.   listed

- allow                                                 		d.   required

Article

- Portable stage and studio lighting equipment and portable power distribution equipment not identified for outdoor use shall be permitted to be permitted to be _____ outdoors if the equipment is supervised by qualified personnel while energized and barriered from the general public.

- used	                                                   	c.   utilized

- arranged                                                     	d.   deployed

Article

- Portable vehicle-mounted and trailer-mounted generator in carnivals, circuses, fairs, and similar events _____ comply with 250.34.

- shall not                                                        	c.  will

- shall                                             		d.  may

Article

- In carnivals, circuses, and fairs the luminaires used for egress lighting shall be _____.

- guarded	                                              	c.   listed

- labeled                                                    	d.   grounded

Article

- An exception to continuous supervision in motion picture and television studios, shall not be required for utility-supplied portable distribution equipment that provides _____ power to other than professional stage and studio equipment, or to production equipment requiring standby power during nonoperation.

- uninterrupted                                            	c.   normal

- standby                                                    	d.   backup

Article

- Generally, all raceways, cables wiring methods and fittings in manufactured buildings shall be _____.

- labeled                                                   	c.   approved

- listed                                                     		d.   identified

Article

- Equipotential planes inside agricultural buildings shall be installed in confinement areas with concrete floors where metallic equipment is located that _____ become energized and is accessible to livestock.

- might                                                             	c.   has

- will                                                         		d.   could

Article

- Ground-fault circuit-interrupter (GFCI) protection in mobile homes, manufactured homes, and mobile homes parks _____ provided as required in 210.8.

- shall not be                                                  	c.   will be

- shall be                                                        	d.   may be

Article

- To determine the voltage converter size in recreational vehicles and parks, percentages to the total connected load of a 12-volt equipment shall have: the first 20 amperes load at 100 percent, plus the second 20 amperes of load at_____ percent plus all loads above 40 amperes at 25 percent.

- 75                                                              	c.  40

- 60                                           			d. 50

Article

- If an inverter or inverter/charger is provided in recreational vehicles and parks, a safety label complying with _____, with the signal word WARNING in minimum ¼ inch high letters and body text I minimum 1/8 inch high letters on a contrasting background.

- 110.21(B)                                       	c.   110.21(A)

- 110.22                                            	d.   110.45

Article

- Recreational vehicles, 120-volt or 12/240-volt, nominal, with a 30- or 50-ampere feeder assembly shall have a listed Type _____ grounding monitor interrupter (GMI).

- I                                             			c.   II

- III                                                      		d.   V

Article

- Where prewiring is installed for the purpose of future inverter installation in a recreational vehicle, devices such as receptacles outlets installed, the installation shall be _____ including circuit conductor connections.

- made-up                                                	c.   capped

- terminated                                                   	d.   complete

Article

- All electrical equipment installed in areas comprising piers, wharves docks, floating buildings, marinas, boatyards, boat basins, boat houses, yacht clubs, or boat condominiums shall be _____..

- listed                                                            	c.  marked

- labeled					d.  identified

Article

- The maximum voltage-ac only for pier power distribution systems shall not exceed _____ volts phase-to-phase.

- 1000	                                                   	c.   480

- 600                                                       		d.   120/240

Article

- The demand factors in Article 120.120 shall be _____ for each service or feeder circuits supplying receptacles that provide shore power for boats.

- allowed                                             		c.  determined

- calculated                                                     	d.  permitted

Article

- All non-current metal parts that are likely to be energized in marines, boatyards, floating buildings, and docking facilities, shall be _____ to the branch circuit or feeder equipment grounding conductor by a bonding conductor.

- attached                                               		c.   connected

- bound                                                     	d.   fixed

Article

- The equipotential plane construction in marinas and boatyards shall have the unencapsulated structural reinforcing steel bonded together by _____ or the equivalent.

- steel tie wire                                               	c.   couplings

- tie wraps                                      		d.   clips

Article

- Equipment in marinas, boatyards, floating buildings, and docking facilities, that has been damaged, shall be recognized, documented, and serviced by a _____ to the edition of this code to which it was originally installed.

- electrician                                                    	c.   maintenance person

- qualified person                                       	d.   electrical engineer

Article

- If shore power is installed to boats, a _____-volt ac, 15- or 20-ampere, GFCI receptacle shall be installed.

- 240                                                		c.   50

- 480                                                     		d.   120

Article

- The exception to the requirement of ground-fault protection of equipment (GFPE) in docking facilities and piers, is applied on the load side conductor of a separately derived system and circuit supplying ground-fault monitoring equipment, that do not exceed _____ and are installed in a raceway shall be permitted without ground-fault protection.

- 10 ft                                                         		c.   6 ft

- 20 ft                                                                	d.   12 ft

Article

- In temporary installations, a guard _____ be required for nonreplaceable LED lamps integral to a listed temporary lighting string.

- shall                                                         	c.   shall not

- will be                                                      	d.   will not

Article

- Temporary installations of cable assemblies, flexible cords, and cables shall not use ______ for support of overhead spans of branch circuits or feeders.

- poles                                                       	c.   vegetation

- straps                                                       	d.   staples

Article

Chapter 6

- Signs with retrofitted illumination systems, shall have _____ that includes the kit provider and installer’s name, logo, r unique identifier.

- marking                                                     	c.   tag

- label					d.   identification

Article

- Outlets for signs shall be supplied by a branch circuit rated at least _____ amperes that supplies no other load.

- 10                                             			c.   25

- 15                                                     		d.   20

Article

- Electric signs installed in fountains or within _____ft of a fountain edge shall use the requirements of Article 680.57.

- 20                                                                 	c.   15

- 10                                                           		d.   50

Article

- Where a crane, hoist, or monorail hoist operates over readily combustible material, the resistors shall be located in a _____ or cab constructed oof noncombustible material that encloses the sides of the cage or cab from the floor top a point at least 6 in. above the top of the resistor.

- box                                        			c.   4-square box

- cabinet                                     		d.   cage

Article

- In elevator hoist ways and pits, Types CL4P, CL4R, and CL4 cable shall be permitted, provided the cables are _____ and protected from physical damage.

- supported                                              	c.   strapped

- bundled                                                     	d.   secured

Article

- The branch circuit for elevator car lighting, receptacle(s), ventilation, heating, and air conditioning shall have the _____ located in the elevator machine room, control room, machine space, or control space.

- breaker                                                      	c.   fuses

- OCPD                                                         	 d.   disconnect

Article

- A single means for disconnecting all ungrounded main power supply conductors for each _____ of an elevator, dumb waiter, escalator, moving walk, platform lift, or stairway chair lift shall be provided and be designed so that no pole can be operated independently.

- system                                                     	c.   location

- unit                                              		d.   control system

Article

- Where elevator system(s) OCPDs are replaced, they shall be _____ to ensure selective coordination is maintained with all supply-side and load-side OCPDs.

- tested               				c.  certified

- re-evaluated     				d.   maintained

Article

- Which Article covers electric self-propelled vehicle power transfer systems (ESVSES)?

- 620                                                   		c.   625

- 630                                                            	d.   624

Article

- ESVSE outlets installed for supplying greater than 16 amperes, or _____ volts, shall be supplied by an individual branch circuit.

- 240                                                       		c.   120

- 24                                                  		d.   208

Article

- Permanently installed electric vehicle power transfer system equipment shall be installed by _____.

- electrician                                                     	c.   factory representative

- qualified person                                      	d.   master electrician

Article

- The output cable to electric vehicles shall be an integral part of listed electric vehicle supply equipment or a _____ type flexible cable as specified in Table 400.4.

- listed                                                    		c.   labeled

- identified                                                    	d.   supply

Article

- EVSE and WPTE including _____ EVSE and WPTE, shall be considered continuous loads.

- multi-directional                               	c.   bidirectional

- unilateral                                 		d.   lateral

Article

- All power control system used to provide _____ management of EVSE shall comply with Article 130 Part II and the maximum equipment load on a service, feeder, and load permitted by the PCS.

- supply                                			c.   power

- vehicle                                   			d.   load

Article

- For multifamily dwellings where multiple EVSE or WPTE are each served through individual disconnecting means, a ______ or directory shall corresponding disconnect clearly identifying which disconnect service which EVSE or WPTE.

- sign                                  			c.   plaque

- label                                   			d.   picture

Article

- Cord- and plug- connected equipment shall be provided with an attachment plug rated at not less than _____ percent off maximum rating of the equipment.

- 125                                                        		c.   200

- 100  					d.   250

Article

- All receptacles installed for the connection of electric vehicle charging shall have ground-fault circuit-interrupter protection for _____.

- equipment                                                     	c.   the vehicle

- personnel                                                 	d.   the unit

Article

- The output cable for EVSE to the primary pad shall be protected from _____ damage and secured in place over its entire length for the purpose of restricting its movement and to prevent strain at the connection point.

- weather                            			c.   physical

- chemical                                                     	d.   external

Article

- Electrified truck parking space equipment shall be _____.

- listed                                                       	c.   marked

- labeled                                                    	d.   identified

Article

- The electrified truck parking space supply equipment shall be provided in:_____.

- pedestal                              			c.   overhead gantry

- raised concrete pad                                   	d.   one of the above

Article

- The electrified truck parking space equipment intended to supply transport refrigerated units (TRU) shall be designed and constructed such that all receptacles outlets supplied by the branch circuits that are rated more than 150 volts to ground, 100 amperes or less, shall be provided with _____ protection for personnel.

- OCPD                                	 		c.   SPGFCI

- specific                                   		d.   GFCI

Article

- The disconnecting means for transport refrigerated units shall be _____, located not more than 750MM (30 in) from the outlet supplying the receptacle or cord connector, and located in the supply circuits ahead of the outlet supplying the receptacle or cord connector.

- accessible                                             	c.   within site

- readily accessible                                   	d.   available

Article

- The power supply cable assembly in transport refrigerated units shall be provided with an integral _____ means that prevents physical damage or rupture, in the event of a drive-away, at the attachment plugs or connectors connection to the respective receptacle or inlet.

- disconnecting                                               	c.   breakaway

- shutoff                                                          	d.   OCPDs

Article

- Audio signal processing, amplification, and reproduction equipment cables shall have tags with which information :_____.

- date cable was identified for future use c.   information related to future use

- date of intended use                                  	  d.   all the above

Article

- The supply circuits for informational technology equipment shall be considered a  _____ load for branch circuits.

- intermittent                                    		c.   semi-continuous

- continuous                                     		d.   part-time

Article

- in information technology equipment where critical operations data system(s) OCPFs are replaced, they shall be re-evaluated to ensure _____ of the critical operations data system(s) is maintained with all supply-side and load-side OCPDs.

- protection                                                   	c.   selective coordination

- coordination                                                 	d.   grounding

Article

- Where flexible power cords or cables are used in modular data centers, they shall be listed as suitable for extra-hard usage and those used outdoors shall also be listed as suitable for wet locations and shall be _____ resistant.

- sunlight                                      		c.   snow

- weather                                             		d.   rain

Article

- Reconditioned x-ray equipment shall _____.

- not be permitted                                  	c.   listed

- be re-evaluated                                  	d.  be permitted

Article

- In swimming pools, fountains, and similar installations where required, ground-fault protection of receptacles and outlets on branch circuits rated 150 volts or less to ground shall be provided with a Class A GFCI with branch circuits rated 60 amperes or less, dingle phase or rated _____ amperes or less, 3-phase.

- 200                                                   		c.   100

- 80                                                           		d.   120

Article

- Fixed or stationary equipment, other than underwater luminaires, for a permanently installed pool shall be permitted to be connected with a flexible cord and plug ant the EGCs shall be connected to the fixed metal part of the assembly and any removeable metal part of the _____ shall be mounted on or bonded to the fixed metal part.

- assembly                                                      	c.   construction

- equipment				d.   conductor

Article

- All electric pool water heaters incorporated resistive heating elements shall have the heating elements subdivided into loads not exceeding _____ amperes ad protected at not over 60 amperes.

- 48                                    			c.   50

- 38                              				d.   45

Article

- In corrosive environments of swimming pools, fountains, and similar installations, _____ wiring methods shall not be permitted.

- copper                                  			c.   copper-clad aluminum

- aluminum                                    		d.   aluminum-clad copper

Article

- Ground-fault protection shall not be installed between the output terminals of the motor drive and the motor for a _____ pool motor employing an adjustable-speed drive.

- multispeed                                                 	c.   single speed

- dual-speed                                                    	d.   variable-speed

Article

- In outdoor pool areas, luminaires, lighting outlets, festoon lighting, and ceiling-suspended (paddle) fans installed above the pool or the area extending 5 ft. horizontally from the inside walls of the pool shall be installed such that _____ are at a height not less than 12 ft. above the maximum water level of the pool

- all parts                                                      	c.   everything

- several parts                                                 	d.   nothing

Article

- The equipotential bonding shall be bonded together using _____.

a.   rigid metal conduit of brass or other     	c.   steel structural welded wire

corrosion-resistant metal			      reinforcement

b.   structural reinforcement steel                 	d.   a, b, or c

Article

- The equipotential bonding of pool parts shall be bonded except:_____.

- metal fittings and parts fixed to             	c.   metallic pool cover anchors intended for

bulkheads constructed of 		       insertion in wood or composite deck

nonconductive material			       surface

- metallic pool cover anchors intended 	d.   all the above

for insertion in concrete or masonry

deck surfaces

Article

- All receptacles located within _____ of the inside walls of a storable pool, storable spa, or stable hot tub shall have GFCI protection or SPGFCI protection.

- 10 ft.                                                      	c.   20 ft.

- 6 ft.                                                      	d.   12 ft.

Article

- In indoor installations, listed spa and hot tub packaged units rated 20 amperes or less _____ permitted to be cord-and-plug-connected to facilitate the removal or disconnection of the unit for maintenance and repair.

- shall not be                               		c.   can be

- shall be                                    		d.   may be

Article

- all receptacles located within _____ of the inside walls of a spa or hot tub shall have GFCI or SPGFCI protection complying with 680.5 as applicable.

- 20 ft.                                                       		c.   4.0 m

- 12 ft.                                                          	d.   10 ft.

Article

- The flexible cord-connected equipment in pools and spas, where the flexible cord is immersed in or exposed to water shall be extra-hard usage type, as designated in ‘’table 400.0 and be a listed type with a “_____” suffix.

- W                                                            		c.   SEW

- S                                                              		d.   STW

Article

- All receptacles located within _____ ft. of a fountain edge shall have GFCI protection or SPGFCI protection, as applicable, in accordance with 680.5.

- 10                                                		c.   20

- 12                          				d.   16

Article

- The branch circuits supply in a hydromassage bathtubs and their associated electrical components shall _____ no other loads.

- supply                                			c.   access

- serve                                			d.   be attached to

Article

- Hydromassage bathtubs and their associated electrical components shall be protected by a _____ GFCI.

- accessible                                  		c.   readily accessible

- special		                                   	d.   designated

Article

- Equipotential planes shall be installed adjacent to all outdoor service equipment or disconnecting means that control equipment in or on water where the system voltage exceeds _____ volts to ground.

- 480                                                         	c.   240

- 1230                                                       	d.   250

Article

- Equipment, such as, PV modules, ac modules, charge controllers, Type PV wire, etc. and any associated retrofit kits, shall be listed or be _____.

- labeled                                                           	c.   identifies

- field evaluated                                           	d.   marked

Article

- The maximum dc voltage, in a solar photovoltaic system, shall be used to determine the voltage and voltage to ground of circuits which exceed _____ volts shall comply with 690.31(G).

- 1000                                                        	c.   600

- 1500                                                        	d.   1200

Article

- The maximum circuit current in a PV source shall be calculated using the sum of the highest short-circuit current rating of the PV modules connected in parallel multiplied by _____.

- 100 %                                                             	c.   125 %

- 70 %                                           		d.   75 %

Article

- The initiation device for rapid shutdown of PV systems on building shall be _____ and consist of one or more as specified in 690.12(C)(1)(a).

- accessible                                                     	c.   readily accessible

- identified                                          		d.   located

Article

- The PV system disconnecting means or their remote operating devices or the enclosures providing access to the disconnecting means shall be _____.

- lockable closed                                  	c.   locked out

- lockable open                                    	d.   lockable

Article

- Manually operable switch or circuit breaker, load-break-rated pull-out switch, or a device listed or approved for the intended application are considered _____.

- listed                                                       	c.   labeled

- recognized                                                   	d.   acceptable

Article

- Class 1 power-limited circuits associated with the PV system or energy management system are _____ to be installed in multiconductor jacketed cable with a jacket insulation rating equal to any conductor installed within the same wiring method.

- not                                                                  	c.   permitted

- can not                                                          	d.   allowed

Article

- PV system dc circuits that exceed _____ volts or 8 amperes shall be provided with ground-fault detector-interrupter protection =.

- 30  		                                  		c.   20

- 50		                                   		d.   100

Article

- Equipment grounding or bonding conductors exposed to cyclical movement in PV arrays with moveable components shall use: _____

- wire-type equipment grounding            	c.   other equipment listed and identified as

conductors				      suitable for the application

- flexible braided straps                             	d.   one or more of the above

Article

- In large-scale PV supply stations the fence _____ and grounding requirements and details shall be included in the documentation required by the engineering design in Article 691.6.

- construction               			c.   bonding

- poles                      				d.   design

Article

- The maximum voltage of fuel cell system dc circuits shall be the highest voltage between any two conductors of a circuit or any conductor and ground and shall comply with: _____.

- fuel cell system dc circuits shall not	c.   neither a or b

exceed 600 volts on or in buildings

- fuel cell system dc circuits shall not      d.   both a and b

exceed 600 volts on or in one- and

two-family dwellings

Article

- Disconnecting means for fuel cell systems, for ungrounded conductors shall consist of readily accessible, _____ operable switches or circuit breakers

- manually                                                    	c.   remotely

- automatically                                              	d.   listed

Article

- In wind electric systems, the overcurrent protection, the _____ side of transformers with sources on each side shall be connected to the largest source of available fault current.

- secondary                                                    	c.   both a and b

- primary                                                    	d.   load

Article

- Fire pump feeder conductors on the load side of the final disconnecting means and where routed through a building, the conductor shall be protected from fire for _____ hours.

- 1	                                                         		c.   2

- 5						d.   4

Article

Chapter 7

- The emergency system equipment shall be suitable for the _____ current at its terminals.

- full load                                                         	c.  fault

- available fault                                          	d.  available

Article

- In emergency systems, a means to bypass the transfer equipment shall be provided while maintaining the continuity of power. Inadvertent _____ operations shall be prevented.

- power		                           		c.   primary

- dual	                                   		d.   parallel

Article

- Emergency system circuits shall be permanently marked and all boxes and enclosures, such as transfer switches, interconnection equipment, generators, and power panels, for emergency circuits shall be _____ marked as a component of an emergency circuit or system.

- clearly		                             		c.   vaguely

- visibly			                        	d.   permanently

Article

- The feeder-circuit wiring in an emergency system shall use cables or raceways listed _____ cable system with a minimum 2-hour fire rating.

- fire-rated                                                       	c.   fire-protective

- fire-resistive                                              	d.   fire-limited

Article

- Powered emergency lighting systems shall have all boxes and enclosure for Class 2 and _____ emergency circuits identified and shall be permanently marked as a component of an emergency circuit or system.

- Class 1		                                  	c.  Class 4

- Class 3		                                   	d.  Class 5

Article

- Equipment for sources of emergency power shall be installed either in spaces fully protected by approved automatic fire protection systems or in spaces with 2-hour fire rating where located within _____.

- buildings above 75 ft in height               	c.   assembly occupancies for more than

1000 persons

- educational occupancies with more      d.   all the above

than 300 occupants

Article

- Where emergency illumination is provided by one or more directly controlled luminaires (DCLs) by ______ of their control input by a listed ELCD upon loss of normal power, such DCLs shall not be required to be listed for use in emergency system.

- disconnection 		                       	c.   direction

- termination		                        	d.   selection

Article

- Class 4 Fault-Managed Power (FMP) transmitters and Class 4 FMP receivers installed as a component of an emergency lighting system shall be _____ as emergency power equipment.

- labeled                                                      	c.   marked

- listed                                                        	d.   identifies

Article

- Selective coordination, in emergency systems, shall be re-evaluated if replacement OCPDs that supply emergency systems loads are not replaced with the same manufacturer and type of OCPDs and with the same rating or _____.

- fault setting                                                 	c.   settings

- manufacturer                                             	d.   values

Article

- The _____ shall conduct of witness the commissioning of the completed legally required standby system upon installation.

- authority having jurisdiction               	c.   engineer

- inspector                                               	d.   building owner

Article

- If the legally required standby loads are supplied by a single feeder, the legally required standby power system shall be permitted to include equipment to facilitate servicing using a means to _____ the transfer equipment shall be provided while maintaining the continuity of power.

- disconnect         				c.   shut-off

- bypass                                                          	d.   remove

Article

- A listed surge protection device shall be installed _____ to or immediately adjacent to all legally required standby system switchgear, switch boards, and panel boards.

- integral                                                       	c.   next

- near                                                       		d.   and permitted

Article

- Legally required standby system power equipment shall be _____ and located so as to minimize the hazards that might cause complete failure due to floods, fires, icing, and vandalism.

- manufactured                              		c.   arranged

- design                                  			d.   built

Article

- Microgrid systems shall be permitted to be connected legally required standby sources and shall include _____ equipment listed for emergency use to isolate the legally required standby system from all non-legally required loads when the normal electric supply is interrupted.

- disconnecting                                              	c.   transferring

- isolating                                               		d.   interconnection

Article

- If OCPDs that supply emergency system loads are not replaced with the same manufacturer and type of OCDPs and with the same rating and settings in legally required standby systems shall have the selective coordination _____.

- reviewed                                                       	c.   portable

- temporary                                  		d.   interlocking device

Article

- Interconnection _____, or transfer equipment shall be required for all optional standby systems subject to the requirements of Article 702.

- disconnects                                  		c.   portable

- temporary		                                   	d.   interlocking device

Article

- In one- and two-family dwelling units, permanently placards, labels, or _____ shall be installed at the disconnecting means or at an approved readily visible location that indicates the location of each permanently installed on-site optional standby power source disconnect or means to shut down the prime mover.

- labels                                 			c.   pictures

- directories                                                    	d.   maps

Article

- Interconnected electric power production source service conductors located in dwelling units and connected to existing service conductors or equipment shall be protected with an _____ located within 10 ft. of conductor length from the point of connection.

- disconnect                                           	c.   OCPD

- transfer switch                                            	d.   service

Article

- Power control systems (PCSs) of the multisource PCS type in interconnected electric power production sources shall be permitted to _____ current loading on busbars and conductors supplied by the output of one or more interconnected electric power production or energy storage sources.

- limit 	                                  		c.   increase

- lower	                                   		d.   shunted

Article

- The disconnecting means for an interconnected electric power production shall plainly_____ whether it is in the open (off) or closed (on) position.

- show                                                          	c.  view

- lower                                                          	d.  indicate

Article

- Interconnected electric power production flexible cord or cable shall be listed and identified as _____cables, or other cords and cables suitable for extra hard use.

- IEPP                                                 		c.  OCPD

- DG                                                         		d.  resistant

Article

- Overcurrent protection in interconnected electric power production, such as circuit breakers not _____ “line” and “load” shall be considered suitable for backfeed.

- labeled		                                  	c.  manufactured

- identified		                                   	d.   marked

Article

- Where ground-fault protection of equipment is installed in accordance with 230.95 or 215.10 and a power source capable of providing ground-fault _____ is installed on the line side, such as ground-fault protection of equipment and any added power source ground-fault protection of equipment shall be performance tested as a system in accordance with 230.95(C).

- interrupter                                  		c.   current

- voltage                                   			d.   equipment

Article

- Article 706 applies to _____ energy storage systems (ESS) having a capacity greater than 1 kWh that could be stand-alone or interactive with other electric power production sources.

- all		                                               	c.  multiple

- several                                                      	d.  every

Article

- When commissioning energy storage systems, the AHJ shall _____ to conduct or witness the commissioning of the ESS.

- be required                                                 	c.  be present

- not be required                                         	d.  not be present

Article

- Listed energy storage systems shall be provided with _____ as required with the listing.

- space	                                                   	c.  disconnects

- current rating                                          	d.  ventilation

Article

- Hazards to be considered for Critical Operations Power Systems (COPS) shall not be limited naturally occurring hazards or _____ events.

- human-caused                                           	c.  man-made

- environmental-caused                              	d.  unforeseen

Article

- A listed _____ shall be installed integral to or immediately adjacent to all critical operations power systems switchgear, switch boards, and panel boards.

- OCPD                                                         	c.  surge protection device

- GFCI                                             		d.  GFPE

Article

- COPS feeders shall be protected against physical damage by: _____.

- rigid metal                                           		c.  Schedule 40 or 80 PVC

- RTRC                                                		d.  all the above

Article

- If the equipment used to bypass the transfer equipment, in COPS, is manual (or nonautomatic), then it shall be actively supervised by a _____ when the primary (automatic) transfer equipment is disabled for servicing.

- engineer                                                        	c.  journeyman electrician

- qualified person                                          	d.  master electrician

Article

- Selective coordination shall be _____  if replacement OCPDs that supply COPS loads are not replaced with the same manufacturer and type of OCPDs and with the same rating or settings.

- re-evaluated                                               	c.  reviewed

- refurbished                                        		d.  revised

Article

- A stand-alone system shall have adequate _____ and rating supply all equipment intended to be operated at one time.

- voltage                                			c.  ability

- current                                                        	d.  capacity

Article

- Which Article covers Limited-Energy System Wiring? _____.

- 750		                                  	c.   720

- 724		                                   	d.   722

Article

- Limited-energy cables located under the pool or area within the area extending _____ from the inside wall of the pool shall meet these requirements specified in 680.11.

- 5 ft.                                                             	c.  10 ft.

- 15 ft.                                           		d.  12 ft.

Article

- Limited- energy system wiring run in both exposed and concealed locations, where a cable-or raceway-type wiring method is installed through bared holes in joist, rafters, or wood members holes shall be bored so that the edge of the hole is not less than _____ from the edges of the wood members.

- 1 ½ in.                                                 	c.  2 in.

- 1 ¼ in.                                              	d.  ¾ in.

Article

- The power source for limited-energy systems for premise-powered broadband communications equipment shall be _____.

- labeled                                                           	c.  listed

- identified                                            		d.  marked

Article

- Article 722: Limited-Energy Cables for Power-Limited Circuits, Fault-Managed-Power Circuits, Optical Fiber Circuits and Communications Circuits, Informational Note No. _____ states that Class4 systems differ from Class1, Class2, and Class3 systems in that they are not limited for power delivered to an appropriate load.

- 3                                                            	c.  2

- 1                                             		d.  4

Article

- Cable routing assemblies for limited-energy systems shall be supported where run horizontally at intervals not to exceed _____ and at each end or joint unless listed for other support intervals.

- 10 ft.                           			c.  5 ft.

- 6 ft.                                           			d.  3 ft.

Article

- Class 4 Fault-Managed Power systems shall have listing information which includes compatible devices if a listed Class 4 device depends on specific system devices for _____, monitoring or control.

- operability                                                                  	c.  inoperable

- interoperability                                                      	d.  guidance

Article

- Overvoltage protection of limited-energy systems shall be permitted to use fuseless primary protectors if conductors enter a building through a cable with grounded _____ sheath member(s) and if the conductors in the cable safely fuse on all currents greater than the current-carrying capacity of the primary protector and of the primary protector bonding conductor or grounding electrode conductor.

- metallic                                         		c.  conductor

- outer                                  			d.  bonding

Article

- For indoor communication purposes only in limited-energy systems, listed sheet metal _____ ground clamps having rigid metal bases that seat on electrodes and have straps of such material and dimensions that they are not likely to stretch during or after installation shall be permitted,

- screw-type                                                  	c.  wire-type

- strap-type                                           		d.  shielded

Article

- The bonding conductor or grounding electrode conductor shall not be smaller than 10 AWG copper, 8 AWG aluminum, or _____ AWG coper-clad steel or bronze for the antenna system and receiving stations in a limited-energy system.

- 12						c.   6

- 14   					d.   17

Article

- The conductors for a non-power-limited fire-alarm shall be _____ AWG or larger solid or stranded copper.

- 16                                     			c.  18

- 10                                     			d.  12

Article

- Transformers or other devices supplied from power-supply conductors in the power-limited fire alarm shall be protected by an OCPD rated not over _____ amperes.

- 20 		                                  		c.   30

- 15                         				d.   10

Article

Chapter 8

- Communication wires, such as, distributing frame wire and jumper wire, for outside and entering buildings, shall be listed as being _____ to the spread of fire.

- shall be                                  			c.   shall not be

- may be		                                  	d.   can be

Article

- Direct-buried premise-powered broadband communications cables shall be installed to have a minimum cover of _____.

- 12 in.                                                              	c.  10 in.

- 18 in.                                                                	d.  6 in.

Article

- Network-powered broadband communications medium power cables shall have insulation for the individual conductors rated for _____ volts minimum.

- 600	                                 		c.   1000

- 300 		                         		d.   1500

Article

- Network-powered broadband communications medium power cables shall be separated at least _____ from conductors of any electric light, power, Class 1, and non-power-limited fire alarm circuits.

- 2 in.                                                          	c.  12 in.

- 6 in.                                                         		d.  18 in.

Article

Annexes

- Annex _____ contains informational references.

- D						c.  B

- C                                        			d.  A

- A new Table _____ Formulas for Mutual Reactance Xm, was added to Annex B.

- B.2(9)					c.  B.2(6)

- B.8                                         			d.  B.2(1)

- Annex D contains two examples for sizing of service conductors for dwelling(s) with or without adjustment for temperature _____ factor(s).

- adjustment				c.  correction

- alternative                                        		d.  modification

- Where is the Load Calculations Using Power Control Systems in Annex D: _____?

- D.4(a)					c.  D.15

- D.5(b)                                        		d.  D.13

- Annex _____ contains informational references.

- Reserved 					c.  Replaced

- Expanded                                        		d.  Illustrated